Quality Assurance Checklist

A checklist is a set of ideas. In testing, checklists come first. They serve as a draft for further activities. Writing and rewriting a checklist is fast and simple. Once the checklist is ready and each item may become the foundation for one or several test cases.

| Project Name | DQ MODULE | Project code | EPM-DQ |
| --- | --- | --- | --- |
| Object Name | Microwave Oven | QA Engineer | Maryna Maksimava |
| Classification by access | Black box – without possibility to explore programing aspects, object components separately or take object apartBlack box – without possibility to explore electronic/software internals or disassemble the appliance |  |  |
| Object description | Overview: A countertop microwave oven for domestic use. Heats food via microwave radiation, features a rotating glass turntable, digital timer/clock display, multi-level power control, preset cooking programs, child lock, and a door safety interlock. Features: Digital keypad, LED/LCD display, auto-cook presets (Popcorn, Defrost), adjustable power levels (300W-1000W), child lock, interior light, turntable with ON/OFF toggle. Model: Samsung MS23K3513AK Web Link Requirements - Rated Voltage: 230 V / 50 Hz - Output Power: 800 W (microwave) + 1150 W (rated input) - Capacity: 23 litres - Maximum Timer: 99 min 99 sec - Power Levels: 5 selectable levels - Noise Level: ≤ 55 dB at 1 metre - Weight: approx. 12 kg - Turntable Diameter: 28.5 cm - Child Lock: Yes - Safety Interlock: Door-stop stops magnetron immediately |  |  |

SMOKE PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Verify power on/off functionality | Positive | No | Functional | Microwave turns on and displays default time (00:00). Power button turns it off. |
| Start basic heating cycle (e.g. 1 min, default power) | Positive | No | Functional | Pressing Start button leads to heating started. Turntable rotates, magnetron hums, timer counts down, stops at 0 with beep. |
| Door open/close and safety interlock | Positive | No | Functional | Door opens and closes without any problems. |
| Door open mid-cycle stops heating | Negative | No | Functional | Opening door during operation immediately pauses the cycle. Heating resumes upon closing and pressing Start. |

CRITICAL PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Timer input – valid numeric entry | Positive | No | Functional | User enters time (e.g. 1:30) via keypad; display shows 1:30; countdown executes correctly. |
| Timer input – invalid / out-of-range values | Negative | No | Functional | Entering 0:00 or exceeding max time (e.g. 99:99) is either rejected or handled. |
| Child lock enable and disable | Positive | No | Functional | Activating child lock prevents all button inputs. Correct unlock sequence restores full function. |
| Heating performance – food reaches target temperature | Positive | No | Performance | After heating a bowl with 200g boiled rice from the fridge for 1:30 it reaches at least 60 °C |
| Stop / Cancel button mid-cycle | Positive | No | Functional | Pressing Stop/Cancel stops the cycle immediately. |

EXTENDED PATH

| Verification area | Test Approach by way of dealing | Would be automated? | Test Type | Expected Result / Comments |
| --- | --- | --- | --- | --- |
| Display readability in light / darkness | Positive | No | Usability | Clock and timer digits are readable in normal room light and in a dimmed room. |
| Defrost mode by weight input | Positive | No | Functional | When the user inputs 300g for defrost, the program runs appropriate duration/power for safe thawing. |
| Noise level during operation | Positive | No | User-Acceptance | Sound level during normal operation does not exceed 55 dB at 1 m distance. |
| Ventilation | Positive | No | Functional | Ventilation starts working with the cycle start and the interior of the microwave is safe to touch as a result of the fan working to remove the excess heat and steam. |